Adjusting the pressurestat (3 wire version)

You will want to do this with the machine off, unplugged, and cool

1) Remove the 5 phillips screws holding the back and sides on (2 on each side, one in the back towards the bottom)

2) unscrew the 3mm allen screws from the left side (near bottom), and also on top of the plate covering the boiler. (Not all machines have the top screws)

3) locate the pressurestat. It is labeled in the attached picture

4) looking downwards at the pressurestat, adjust 1/4 turn counter-clockwise to lower pressure, or clockwise to increase pressure. If more adjustment is needed, keep turning 1/4 turns at a time.

Once 1.2-1.4bars has been reached, or your relief valve stops releasing steam, your pressure has been adjusted.

Should you need further assistance, please call 1-888-411-5282 x3 weekdays from 9am-5pm eastern time.